- Reading the Input file

- Initialization

  - Initialize GLOBAL_History  to 11 as strong taken

  - Initialize Local variable array to 00

  - Initialize Global variable array to 11 as taken

  - Initialize slector variable (Seltr) to 01 ;

  - Also initialize Seltr_Dir, Right and wrong prediction counter for local, global, final tournament prediction, hit, collision and wrong address.

- Generating BTB pointer and checking for branch in trace (Flag_trace_Jump)  FlagA

- Check if a current branch was in BTB before (Flag_PC_In_BTB)  FlagB

Checking the Branch Status

- Flag A = If we have a jump

- Flag B = If PC is in the BTB

A Taken Branch which is in the table as well

- IF Flag A = 1 and Flag B = 1   		(A Branch which is taken)

  - Hit = Hit+1

  - Set the Final Predictions

  - Set the Selector and Direction.

  - If Target is in the BTB

    - Set Local Predictor

    - Set Global History

    - Set Global Predictor

  - Else	(Target is not in the BTB)

    - Wrong Prediction Local + 1

    - IF (Current PC In sample file is not equal to PC in BTB and it is not the first time (not zero))

      - Collision + 1

    - Put the Current PC into the BTB

    - Put the Target PC into the BTB

    - Set the Local Predictor = 11

    - Set the Global History

    - Set the Global Predictor = 11

A Branch which is not taken

- ElseIF FlagA = 0 and Flag B = 1   		(A Branch which is not taken)

  - Set the Final Predictions

  - Hit = Hit+1

  - Set the Selector and Direction.

  - Set the Global History

  - Set the Global Predictor

  - Set the Local Predictor

A Branch which is taken for the first time

- ElseIF FlagA = 1 and Flag B = 0   		(A Branch which is taken for the first Time)

  - Set the Final Predictions

  - Set the Selector and Direction.

  - IF (Current PC In sample file is not equal to PC in BTB and it is not the first time (not zero))

    - Collision + 1

  - Put the current PC into the BTB

  - Put the target PC into the BTB

  - Set the local Predictor

  - Set the Global History

  - Set the Global Predictor

  - Branch Miss Counter + 1

Not Branch at all

- Else If FlagA =0 and FlagB = 0 		(A Normal Command)

If Seltr_Dir = 10

If (Sletr = 10)

Seltr = 11

elseIf (Sletr = 01)

Seltr = 10

elseIf (Sletr = 00)

Seltr = 01

Else

Seltr = Seltr

End

elseIf Seltr_Dir = 01

If (Sletr = 10)

Seltr = 01

elseIf (Sletr = 01)

Seltr = 00

elseIf (Sletr = 00)

Seltr = 00

Elseif (Sletr = 11)

Seltr = 10

End

Else

Seltr = Seltr

end