Project 4 EE596

Review of the paper 21.3 ISSCC 2019

Mohammad Chahardori

WSU ID : 11632753

Date: April 29

This paper presents the design and implementation of RF front-end section of a beam former which has been designed for 5G application. The paper includes some new ideas in system level and circuit level design which I describe and summarize some of them in this document.

- System level techniques and novelties:

The proposed system gains benefits of Fully Connected (FC) Hybrid Beam Former (HBF) as appose to conventional Partially Connected (PC) HBF. Fig.1 shows the structure of FC-HBF and PC-HBF together. In PC-HBF each antenna in the antenna array is connected to one beamforming signal path and each antenna belongs to one stream of the RX/TX signal, while in FC-HBF each antenna is connected to one of the RX/TX beamforming signal path of each stream of RX/TX signal.

Fig.1. Basic structure of PS and FC HBF.

By beamforming signal path, I mean the signal path which includes the phase shifter and amplitude weighting blocks. As a result, in FC-HBF each antenna element is connected to Ns signal path at the same time where Ns is the number of streams in one TRX chip. The FC-HBF structure has some advantage and one major disadvantage over PC-HBF which are mentioned as follow:

- Advantages:

- FC-HBF has better spectral efficiency. In fact, in FC-HBF to transmit or receive several streams of signal concurrently, we are using all of the antenna elements at the same time while in the PC-HBF each stream exploits only part of the antenna elements.

- FC-HBF is more energy efficient compare to PC-HBF especially in TX mode. To describe this point, we can examine and compare the required power and the efficiency of the PAs in FC and PC structure.  To have same amount of radiated power at the antenna point (Same EIRP) in PC each stream send power to N/Ns antennas where N is the total number of antennas in the whole antenna array. As a result, the average power of the PC structure is:

While in FC each stream send power to all N antennas at the same time. As a result, the average power of the FC structure is:

This calculation shows that to send same amount of radiated power to the antenna, FC requires less average output power compared to PC. The ratio of average power in these two cases can be calculated as:

Based on this equation we can find that the required average power in FC system is 1/NS times less than average power in PC system. On the other hand, because of the structure of FC and PC systems we know that each PA in FC system should transmit power to NS stream while each PA in PC system should transmit power to only one stream. That's means the required power in FC system is NS time higher than the power in PC system. That is  . Considering these two relation between the generated power by FC and PC systems we can realize that both systems have same peak power. The derived equations show that the FC system needs less average power compared to PC system with the same peak power, so if we use non class-A structure (Class-A Pas consume specific average power regardless of the input/output signal average power) as PA we would have better Energy Efficiency in whole system with FC structure.

- Capability of FC system for self-interface cancellation.

- disadvantages:

The main disadvantage of FC compare to PC is the complexity of FC. In FC system we need to connect each antenna to several RX/TX path while in PC each antenna element is connected to one signal path.

- System/Circuit level techniques which are used in the proposed structure:

Another method which has been used in this structure is using a star separate antenna connection instead of shared antenna connection.  This method improves the isolation between transmitter and receiver and reduce the self-interfering phenomenon. Also in this case, we don't need to use circulator since we have two separate signal paths for TX and RX.

As mentioned before the proposed structure also gained benefits from self-interface cancellation mechanism using the TX and RX paths at the same time to cancel self-interference. In fact, FC HBF structure inherently supports per-element RF-domain self-interface cancellation which is one of the benefits of the Proposal structure.

- Circuit level techniques which are used in the proposed structure:

The most important circuit level technique that is introduced in the proposed circuit is its dual-band bi-directional front-end structure which can be used in three different modes of operation. (TX, RX and Full-Duplex FDD modes). In fig.2 the bi-directional structure is shown.

Fig.2. Proposed bi-directional structure and its different fundamental blocks

Some circuit techniques and modification have been employed in this structure which are described here:

- Multi-band antenna interface network:

As shown in fig.2 the proposed structure used several transformers to connect both receiver and transmitter to the antenna. They have used switches S1 to S4 to realize impedance matching in both bands when TX or RX part is not working. In RX mode when Pas are off S3 and S4 are on (shorted) which reduce input path inductance and cause wideband input matching when the LNA structure is a common gate amplifier. During TX S1 is on and remove the input impedance of the LNA also if one of the PA in low power transmitting mode is of its corresponding switch would be on and remove the off PA effect. All other switches connected at the output of Pas are off to let Pas work properly.

- Power combining PA structure:

Each TX path has two slices. The output transformers in fig.2 also are used to combine the power of the PAs at the end of two TX slices to generate the output power.

- Multi stage amplifier in TX:

Each slice consists of three drivers and one output Class-B PA.  They have used common mode and differential neutralization in amplifiers and dual band inter-stage matching.

- Using proper capacitor at the output of transformer to generate Dual-Band 2nd harmonic short as seen in fig.3. As we can see in this figure the transformer and capacitors 2C generate a forth order impedance which has two zeros at f1 and f2. By adjusting the values of 2C and inductance of the transformer they are generating short circuit at 2nd harmonics of desired signal to reduce these 2nd harmonics.

- They have used self-neutralized structure but they have not descried it in this paper. They referred the method to one of their old publication.

Fig.3. Dual-Band 2nd harmonic short structure